# Tab 2

Creating a project

- Go to “Project > New Project” and choose a name and file location

- Select the device as shown below

- At “Device Settings” page, use the defaults and click “Next”

- “Add HDL Sources” will allow you to start the project with existing HDL source files, but HDL source files can also be added later.

- We will not use “Add Constraints” during project creation.

Making a SmartDesign

- Select “File > New > SmartDesign”

- Name your SmartDesign.

- Right-click your SmartDesign and click “Set as Root”

Adding source files to the SmartDesign

- Open the folder “project_name > hdl” in Visual Studio Code (Lookup how to install VSCode if not already installed)

- In Visual Studio Code, make a .sv file in the “hdl” folder by right clicking

Here is a sample SV file to test:

module blinky( //a module is kind of like a function in other coding languages. This module's name is "blinky"

input logic pushButton, //input logic for input signals (pushButton is the variable (wire) name)

output logic light //output logic for input signals (light is the variable (wire) name)

); //End of port definitions (inputs/outputs)

assign light = pushButton; //this connects the input variable to the output variable

//with a physical wire, hence the term "Hardware Description Language"

endmodule //End of the blinky module

- In Libero, click “Build Hierarchy” for Libero to recognize the new source file.

  - Libero will only recognize source files that have modules, meaning that the .sv file must have “module…endmodule”

- Access the SmartDesign by double clicking it in the “Design Hierarchy” tab

- In the SmartDesign, drag the module “blinky” onto the workspace to see its ports.

- Additional source files can be added to the folder and dragged onto the workspace.

Adding top-level input/output ports in the SmartDesign

- Right-click the workspace and select “Add Port”

- Specify input/output and the bit depth ([N-1:0] for N bits)

- You can drag to connect a port.

Synthesis

- In your SmartDesign, click the “Generate Component” icon (looks like a yellow barrel)

- Under the “Design Flow” tab, double-click “Synthesize” (under “Implement Design”)

Pin Assignment

- Under the “Design Flow” tab, double-click “Manage Constraints” (under “Constraints”)

- Under “I/O Attributes” click “Edit > Edit with I/O Editor”

- Assign each “Pin Number” as shown below (also available in the excel sheet)

- In the top left, click the blue commit icon.

- Close the I/O editor

Place and Route

- Under the “Design Flow” tab, double-click “Place and Route” (under “Implement Design”)

Flash to the FPGA

- Connect your PC to the FPGA using a USB-C cable

- Under the “Design Flow” tab, scroll down and double-click “Run PROGRAM Action” (under “Program Design”)

- Wait for the FPGA to flash. Once it is done, the left-most LED should turn off when you press push button 1! It is on by default because the buttons are active-low, meaning a logical 0 is represented by a button press and logical 1 by default. Now you are ready to implement more complicated designs!